| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO: 01

Objective:

- Introduction to the Micro – Controller and their understanding.

- Understanding of the software code for the blinking of LED.

Introduction:

A microcontroller is a compact integrated circuit designed to govern a specific operation in an embedded system. A typical microcontroller includes a processor, memory and input/output (I/O) peripherals on a single chip.

Sometimes referred to as an embedded controller or microcontroller unit (MCU), microcontrollers are found in vehicles, robots, office machines, medical devices, mobile radio transceivers, vending machines and home appliances among other devices.

Even at a time when Intel presented the first microprocessor with the 4004 there was already a demand for microcontrollers: The contemporary TMS1802 from Texas Instruments, designed for usage in calculators, was by the end of 1971 advertised for applications in cash registers, watches and measuring instruments. The TMS 1000, which was introduced in 1974, already included RAM, ROM, and I/O on-chip and can be seen as one of the first microcontrollers, even though it was called a microcomputer. The first controllers to gain really widespread use were the Intel 8048, which was integrated into PC keyboards, and its successor, the Intel 8051, as well as the 68HCxx series of microcontrollers from Motorola.

Microcontroller Features:

A microcontroller's processor will vary by application. Options range from the simple 4-bit, 8-bit or 16-bit processors to more complex 32-bit or 64-bit processors. In terms of memory, microcontrollers can use random access memory (RAM), flash memory, and EEPROM. Generally, microcontrollers are designed to be readily usable without additional computing components because they are designed with sufficient onboard memory as well as offering pins for general I/O operations, so they can directly interface with sensors and other components.

Some other Features are:

- Digital I/O Pins

- Analog I/O Pins

- Interrupts

- Timers

- Power Consumption and Sleep

Microcontroller Applications:

Today, microcontroller production counts are in the billions per year, and the controllers are integrated into many appliances we have grown used to, like

- household appliances (microwave, washing machine, coffee machine, . . . )

- telecommunication (mobile phones)

- automotive industry (fuel injection, ABS, . . . )

- aerospace industry

- industrial automation

- . . .

The simplest microcontrollers facilitate the operation of electromechanical systems found in everyday convenience items, such as ovens, refrigerators, toasters, mobile devices, key fobs, video games, televisions and lawn-watering systems. They are also common in office machines such as photocopiers, scanners, fax machines and printers, as well as smart meters, ATMs and security systems.

TM4C123GH6PM (ACTIVE):

High performance 32-bit ARM® Cortex®-M4F based MCU.

The TM4C123GH6PM microcontroller is targeted for industrial applications, including remote monitoring, electronic point-of-sale machines, test and measurement equipment, network appliances and switches, factory automation, HVAC and building control, gaming equipment, motion control, transportation, and fire and security.

Now we are going to understand how to code the TM4C123GH6PM using the Energia software. This software is same interface as Arduino’s IDE. Here we use the same setup function for initialization of the pins and loop function for the repeated program. The following fig shows the coding interface of Energia.

Interface of Coding Environment for TIVA Microcontroller

Task 1: Energia Code For LED Blinking:

Task 2: All Three LED’s

Conclusion:

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 02:

Blinking LED using Switch

Objective:

- Comprehension of switch functioning in Micro-controller.

- Able to write the code for switching led.

Apparatus:

- TIVA C Board

- USB Cable

- PC

- ENERGIA IDE

Theory:

TM4C123 micro-controller is a multi-functional device. Microcontroller on the board is basically a debugger that check the preciseness and correctness of program. If there is any error in code it is detected by debugger and alarming message comes to screen. In this lab we will use different peripherals of board to gain desired results. Board contain RGB led and user switches. There is also a reset key on board to reset the code. Here is simple circuit of RGB with switches that we will use in lab.

Figure 1RGB circuit with switches

Procedure:

- Set up the experiment as shown in the figure 2.1.

- Connect TIVA C Board to PC using USB Cable.

- Now write code for blinking of led by using single switch.

- Then, upload this code to TIVA C Board.

- Repeat this experiment for different tasks.

Figure 2Tiva tm4c123 Board

Lab Exercises:

Task 1.1: Blink single led by using switch.

Task 1.2: Blink two led by using two separate switches.

Task 1.3: Blink two led by using single switch (one after the other).

Conclusion:

how to get our desired result using very simple coding in microcontroller.

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 03:

Analog Input

Objective:

- Understanding of PWM Analog Signal and Analog Input from Sensor.

- Able to write the code for Analog Input.

Apparatus:

- TIVA C Board

- ENERGIA IDE

- Potentio – Meter

- USB Cable

- PC

Theory:

TM4C123 micro-controller is a multi-functional device. In order to fade your LED off and on, gradually increase your PWM value from 0 (all the way off) to 255 (all the way on), and then back to 0 once again to complete the cycle. In the sketch below, the PWM value is set using a variable called brightness. Each time through the loop, it increases by the value of the variable fadeAmount.If brightness is at either extreme of its value (either 0 or 255), then fadeAmount is changed to its negative. In other words, if fadeAmount is 5, then it is set to -5. If it’s -5, then it’s set to 5. The next time through the loop, this change causes brightness to change direction as well. analogWrite() can change the PWM value very fast, so the delay at the end of the sketch controls the speed of the fade. We can also read input from sensor in order to calculate voltage. For this purpose rheostat is used which provide resistance to the flow of voltage.

PWM Analog Input:

Procedure:

- Connect TIVA C Board to PC using USB Cable.

- Now write code for PWM analog input for fading effect.

- Then, upload this code to TIVA C Board.

- Now check the result.

Lab Exercises:

Task 1.1: Control LED by using PWM analog signal.

Analog Input Read from Sensor:

Procedure:

- Do setup as shown in figure 1.1.

- Potentiometer attached to analog input.

- Center pin of the potentiometer to the analog pin A2.

- One side pin (either one) to ground.

- The other side pin to +3.3V.

- Build in LED is controlled by sensing analog input.

Figure 1: Analog sensor input

Lab Exercises:

Task 1.2: Read analog input from analog sensor.

Conclusion:

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 04:

Simulation of Code in Assembly Language

Objective:

- Introduction to Assembly Language.

- Using Keil MDK software to implement assembly language and debug our program.

- Understanding of the Assembly language and Format with first assembly code.

Apparatus:

- Tm4c123 microcontroller board.

- Kiel uVision software.

Introduction of Assembly Language:

In computer programming, assembly language (or assembler language), often abbreviated asm, is any low-level programming language in which there is a very strong correspondence between the instructions in the language and the architecture's machine code instructions. Assembly language may also be called symbolic machine code.

Assembly code is converted into executable machine code by a utility program referred to as an assembler. The conversion process is referred to as assembly, as in assembling the source code. Assembly language usually has one statement per machine instruction (1:1), but comments and statements that are assembler directives, macros, and symbolic labels of program and memory locations are often also supported.

Assembly language uses a mnemonic to represent each low-level machine instruction or opcode, typically also each architectural register, flag, etc. Many operations require one or more operands in order to form a complete instruction. Most assemblers permit named constants, registers, and labels for program and memory locations, and can calculate expressions for operands. Thus, the programmers are freed from tedious repetitive calculations and assembler programs are much more readable than machine code. Depending on the architecture, these elements may also be combined for specific instructions or addressing modes using offsets or other data as well as fixed addresses. Many assemblers offer additional mechanisms to facilitate program development, to control the assembly process, and to aid debugging.

Advantages of Assembly Language:

Having an understanding of assembly language makes one aware of −

- How programs interface with OS, processor, and BIOS;

- How data is represented in memory and other external devices;

- How the processor accesses and executes instruction;

- How instructions access and process data;

- How a program accesses external device

Keil MicroVision:

Arm Keil MDK is the most comprehensive software development solution for Arm-based microcontrollers and includes all components that you need to create, build, and debug embedded applications.

KEIL uVision IDE. KEIL is an integrated development environment created by ARM, which includes C/C++ compiler, a text editor called uVision and a sequence of debugging tools for various ARM-based microcontrollers.

Based on the µVision® IDE, it combines project and run-time environment management, as well as build facilities with source code editing and program debugging in a single environment. It provides leading support for Arm Cortex-M based devices, including the Armv8-M architecture (Cortex-M23/M33/M35P).

Embedded system means some combination of computer hardware and programmable software which is specially designed for a particular task like displaying message on LCD. If you are still wondering about an embedded system, just take a look at these circuit applications using 8051 microcontroller. You can call these applications embedded systems as it involves hardware (8051 microcontroller) and software (the code written in assembly language).

Figure 1 Interface of Keil MicroVision

Procedure:

- First we have to install keil software.

- Connect the Tiva C board with computer.

- Connect the board with software internal setting.

- Create a new project file for writing a program.

- Write our first assembly code

Debug the code and execute it.

Task: Write an Assembly code to get the sum of first 5 even numbers:

Code:

THUMB ; Marks the THUMB mode of operation

StackSize EQU 0x00000100 ; Define stack size of 256 byes

AREA STACK, NOINIT, READWRITE, ALIGN=3

MyStackMem SPACE StackSize

AREA RESET, READONLY

EXPORT __Vectors

__Vectors

DCD MyStackMem + StackSize ; stack pointer for empty stack

DCD Reset_Handler ; reset vector

AREA MYCODE, CODE, READONLY

ENTRY

EXPORT Reset_Handler

Reset_Handler

Keil Output:

Result:

R0 is initialized with 0,R1 with 2 (first even number) and R2 with 5 (for five iterations). First CBZ command check the value in R2 and compare it with zero if condition is true it then goes to next label if false then it goes to next statement. Initially, 5 is stored in R2 so Keil executes next statement i.e. adds 2 which is stored in R1 into R0 then there is decrement in the value of R2 and the Command B denotes branching statement and loop repeats itself five times when value in R2 becomes zero then on being compared with zero in CBZ statement it goes to next label and program terminates. We can finally observe that final value in R0 is 0x0000001E which is equal to 30 (Decimal). Sum = 2+4+6+8+10 = 30 (Hence Proved). That sum of first five even numbers is thirty.

Conclusion:

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 05:

Assembly Language Instructions

Objective:

To understand TIVA-C-TM123G Microcontroller’s working and performing three basic examples of add numbers using TM4C123G Launchpad kit on Keil Software. Main objectives of lab are as follow:-

- Using AND ORR and shifting commands.

- Solve y = 64x + c

- Using MLA MLS commands solve equations:

Apparatus:

- Tm4c123 microcontroller board.

- Kiel uVision software.

Introduction:

An assembly language is a low-level programming language designed for a specific type of processor. It may be produced by compiling source code from a high-level programming language (such as C/C++) but can also be written from scratch. Assembly code can be converted to machine code using an assembler.

Since most compilers convert source code directly to machine code, software developers often create programs without using assembly language. However, in some cases, assembly code can be used to fine-tune a program. For example, a programmer may write a specific process in assembly language to make sure it functions as efficiently as possible.

While assembly languages differ between processor architectures, they often include similar instructions and operators.

Task 1-Using AND, ORR, BIC, SHIFTING:

THUMB;

StackSize EQU 0x00000100;

AREA STACK, NOINIT, READWRITE, ALIGN=3

MyStackMem SPACE StackSize

AREA  RESET, READONLY

EXPORT __Vectors

__Vectors

DCD MyStackMem + StackSize ;

DCD Reset_Handler

AREA MYCODE, CODE, READONLY

ENTRY

EXPORT Reset_Handler

Reset_Handler

TASK 2: Solve y = 64x + c

THUMB;

StackSize EQU 0x00000100;

AREA STACK, NOINIT, READWRITE, ALIGN=3

MyStackMem SPACE StackSize

AREA  RESET, READONLY

EXPORT __Vectors

__Vectors

DCD MyStackMem + StackSize ;

DCD Reset_Handler

AREA MYCODE, CODE, READONLY

ENTRY

EXPORT Reset_Handler

Reset_Handler

Task3-Using MLA MLS command solve equations:

THUMB;

StackSize EQU 0x00000100;

AREA STACK, NOINIT, READWRITE, ALIGN=3

MyStackMem SPACE StackSize

AREA  RESET, READONLY

EXPORT __Vectors

__Vectors

DCD MyStackMem + StackSize ;

DCD Reset_Handler

AREA MYCODE, CODE, READONLY

ENTRY

EXPORT Reset_Handler

Reset_Handler

Conclusion:

In this lab our main task was to practice some commands of assembly language that we have learned in our theory and observe their output on the KEIL MICRO – VISION Software and in addition to these tasks we also solved some equations using these commands and observed their outputs. And in the last we concluded that we can write some codes more efficiently by knowing that which command take more clock cycles and which one take less and using those commands which can do the same job but with less clock cycles.

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 06:

GPIO configuration in Assembly & C Language on TM4C123 Board

Objective:

- How external devices are configured.

- Different steps involved in configuration.

- Logic for applying the code.

- Understanding of Code.

- Which operation is required for toggling

Apparatus:

- Tm4c123 microcontroller board.

- Kiel uVision software.

Overview:

An assembly language is a low-level programming language designed for a specific type of processor. It may be produced by compiling source code from a high-level programming language (such as C/C++) but can also be written from scratch. Assembly code can be converted to machine code using an assembler.

Since most compilers convert source code directly to machine code, software developers often create programs without using assembly language. However, in some cases, assembly code can be used to fine-tune a program. For example, a programmer may write a specific process in assembly language to make sure it functions as efficiently as possible.

While assembly languages differ between processor architectures, they often include similar instructions and operators.

Toggling of Led using Assembly Language:

In this Lab our main objective is to write a code that can be able toggle the led with desired delay we provide with so for this purpose we will be using the KEIL MDK software to write the assembly language code that we do all the work on the hardware.

And our next goal is to do the same thing using the C language. And we will how our code become smaller because we will able to perform the “Read Modify Write” method very easily using just one single command instead of many commands as compare to in assembly language.

Task # 1: Led’s Code in Assembly Language

THUMB ; Marks the THUMB mode of operation

; Constants are declared in CODE AREA

AREA Constdata, DATA, READONLY

; Defining register addresses and constants

SYSCTL_RCGCGPIO_R		EQU	0x400FE608

GPIO_PORTF_DATA_R		EQU	0x40025020

GPIO_PORTF_DIR_R		EQU	0x40025400

GPIO_PORTF_AFSEL_R		EQU	0x40025420

GPIO_PORTF_DEN_R		EQU	0x4002551C

GPIO_PORTF_CLK_EN		EQU	0x20

GPIO_PORTF_PIN3_EN		EQU	0x08

SYSTEM_CLOCK_FREQUENCY	EQU	16000000 ; Default clock

DELAY_VALUE EQU SYSTEM_CLOCK_FREQUENCY /80

GPIO_PORTF_PIN3 			EQU 	0x08 ; For green LED

;	The user code ( program ) is placed in CODE AREA

AREA |. text |, CODE , READONLY , ALIGN =2

ENTRY ; ENTRY marks the starting point of

EXPORT __main

; User Code Starts from the next line

__main

Task # 2: Toggling of Led’s using C language:

#define SYSCTL_RCGC2_R *( ( volatile unsigned long *) 0x400FE108 )

#define SYSCTL_RCGC2_GPIOF 0x00000020

#define GPIO_PORTF_DATA_R *(( volatile unsigned long *) 0x40025020 )

#define GPIO_PORTF_DIR_R *(( volatile unsigned long *) 0x40025400 )

#define GPIO_PORTF_DEN_R *( ( volatile unsigned long *) 0x4002551C )

#define SYSTEM_CLOCK_FREQUENCY 16000000

#define DELAY_VALUE SYSTEM_CLOCK_FREQUENCY /80

#define GPIO_PORTF_PIN3 0x08

unsigned long j =0;

int main( )

{

SYSCTL_RCGC2_R|= SYSCTL_RCGC2_GPIOF;

GPIO_PORTF_DEN_R|= GPIO_PORTF_PIN3;

GPIO_PORTF_DIR_R|= GPIO_PORTF_PIN3;

while(1)

{

GPIO_PORTF_DATA_R^= GPIO_PORTF_PIN3;

for ( j =0; j < DELAY_VALUE; j++);

}

}

Output:

Conclusion:

In this lab we further increased our study in assembly language and wrote our first code for the toggling of led and then we performed the same task in the C language using the same software. And in this case we observed that the code for C language was shrunk so the same job of read modify and write was done by the compiler itself. Overall it was great experience to learn.

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 07:

GPIO Switch and LED Code in C Language on TIVA-C Board

Objectives:

To understand TIVA-C-TM123G Microcontroller’s working and performing TM4C123G Launchpad kit on Keil Software. Main objectives of lab are as follow:-

- Understanding of Switch Control.

- Understanding of Edge control and level control.

- Understanding the Delay_Debounce function.

Apparatus:

- Tm4c123 microcontroller board.

- Kiel uVision software.

Overview:

An assembly language is a low-level programming language designed for a specific type of processor. It may be produced by compiling source code from a high-level programming language (such as C/C++) but can also be written from scratch. Assembly code can be converted to machine code using an assembler.

Since most compilers convert source code directly to machine code, software developers often create programs without using assembly language. However, in some cases, assembly code can be used to fine-tune a program. For example, a programmer may write a specific process in assembly language to make sure it functions as efficiently as possible.

While assembly languages differ between processor architectures, they often include similar instructions and operators.

Application of Switch by controlling of Led:

Mechanical switches are commonly used to feed any parameters to the digital systems. The switches can be interfaced to a microcontroller using digital inputs. The software program for switch interfacing can be implemented using one of the following two methods.

- Polling Based: In case of polling based method the GPIO pin connected with the switch is polled frequently enough, in the software, to avoid missing any key presses. By frequently enough, it is meant that the time interval between two consecutive read operations of the GPIO port pin connected with the switch and determining whether the switch is pressed or not, is smaller than the minimum time the switch is kept pressed by the user.

- Interrupt Based: In this method the GPIO pin is configured as an external interrupt and any key press leads to an interrupt. The edge triggering of the interrupt can be configured on rising or falling edge, depending on the switch hardware connectivity.

Switch Bouncing

Electrical switches that use mechanical contacts to close or open a circuit are subject the bouncing of the contacts. Switch inputs are asynchronous and are not electrically clean. Switch contacts are usually made of springy metals. When the contacts strike together, their momentum and elasticity act together to cause them to bounce one or more times before making steady contact. It results in a pulsating electric signal instead of a clean transition. For a microcontroller GPIO pin configured as input, this switch bouncing can be interpreted as multiple switch presses. Figure shows the bouncing of a switch with normally open contact and is captured using digital storage oscilloscope.

Both hardware as well as software solutions exist to get around the switch bouncing problem. Analog filtering using an RC delay to filter out the rapid changes in switch output can be used as hardware solution. The design task is to choose R and C such that the input threshold is not crossed while bouncing is still occurring. In case of software solution to the switch bouncing problem, at the beginning of polling the switch is checked for being pressed. If the switch is detected as pressed, then a delay (corresponding to switch debouncing interval) is inserted and the switch is checked again for being pressed. If the switch is detected as pressed again, we declare that the switch is pressed. The rate at which the switch should be polled has to be fast enough that no switch press is missed, i.e., the time gap between two consecutive switch presses is larger than the scan cycle duration.

Switch Interfacing Illustration:

There are two user switches on TM4C123 evaluation board. User switch labeled SW1 is connected to Port F pin 4 (abbreviated as PF4), while the second switch SW2 is connected to Port F pin 0 (PF0). In the following, we will describe the configuration steps required for interfacing switch SW1. The basic configuration steps are similar to the LED interfacing, with the exception of port pin direction configuration, as listed below:

- Enable the clock to the GPIO port.

- Select the general purpose IO functionality by clearing the alternate function select register (GPIOAFSEL).

- Configure the GPIO direction for digital input.

- Configure the PAD for digital operation. In addition, enable the internal pull-up resistor by setting the corresponding bit of GPIOPUR register. This is required for proper functioning of the switch, as no external pull-up resistor is placed on the board.

Task: Code for LED’s control using Switch:

# define SYSCTL_RCGC2_R *

(( volatile unsigned long *)0 x400FE108 )

# define SYSCTL_RCGC2_GPIOF 0x0020 // Port F clock gating control

# define GPIO_PORTF_DATA_RD *

(( volatile unsigned long *)0 x40025040 )

# define GPIO_PORTF_DATA_WR *

(( volatile unsigned long *)0 x40025020 )

# define GPIO_PORTF_DIR_R *

(( volatile unsigned long *)0 x40025400 )

# define GPIO_PORTF_DEN_R *

(( volatile unsigned long *)0 x4002551C )

# define GPIO_PORTF_PUR_R *

(( volatile unsigned long *)0 x40025510 )

# define GPIO_PORTF_PIN3 0x08

# define GPIO_PORTF_PIN4 0x10

// Default clock frequency

# define SYSTEM_CLOCK_FREQUENCY 16000000

# define DEALY_DEBOUNCE SYSTEM_CLOCK_FREQUENCY /1000

// Function prototypes

void Delay ( unsigned long counter );

// User main program

int main ()

{

static char flag = 0;

// Enable the clock for port F

SYSCTL_RCGC2_R |= SYSCTL_RCGC2_GPIOF ;

// Configure PF3 as digital output and PF4 as digital input

GPIO_PORTF_DEN_R |= GPIO_PORTF_PIN3 + GPIO_PORTF_PIN4 ;

GPIO_PORTF_DIR_R |= GPIO_PORTF_PIN3 ; // PF3 as output

GPIO_PORTF_DIR_R &= (~ GPIO_PORTF_PIN4 ); // PF4 as input

GPIO_PORTF_PUR_R |= 0 x10; // Enable pull up on PF4

while (1)

{

if( GPIO_PORTF_DATA_RD == 0)

{

Delay ( DEALY_DEBOUNCE );

if (( flag == 0) && ( GPIO_PORTF_DATA_RD == 0))

{

// Toggle the green LED

GPIO_PORTF_DATA_WR ^= GPIO_PORTF_PIN3 ;

flag = 1;

}

}

else // clear the flag only when the switch is released

{

flag = 0;

}

}

}

/* This function implements the delay */

void Delay ( unsigned long counter )

{

unsigned long i = 0;

for (i =0; i< counter ; i ++) ;

}

Conclusion:

In our previous lab we had learned how toggle the led by using the function EOR now in this lab we just extended our study by adding a switch control to led so we control the led by using the press of a push button. And we the edge trigger method in which we are reading the edge of a signal and we also used the function delay_debounce which helps us to remove the noise generated by the switch itself.

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 08:

7-Segment Display Control using the TIVA-C Board

Objectives:

To understand TIVA-C-TM123G Microcontroller’s working and performing TM4C123G Launchpad kit on Keil Software. Main objectives of lab are as follow:-

- Understanding of Switch Control.

- Understanding of Time Multiplexing.

- Understanding the Delay_Debounce function.

Apparatus:

- Tm4c123 microcontroller board.

- Kiel uVision software.

Overview:

An assembly language is a low-level programming language designed for a specific type of processor. It may be produced by compiling source code from a high-level programming language (such as C/C++) but can also be written from scratch. Assembly code can be converted to machine code using an assembler.

Since most compilers convert source code directly to machine code, software developers often create programs without using assembly language. However, in some cases, assembly code can be used to fine-tune a program. For example, a programmer may write a specific process in assembly language to make sure it functions as efficiently as possible.

While assembly languages differ between processor architectures, they often include similar instructions and operators.

Seven-Segment LED Interfacing:

The seven-segment display is one of the widely used displays that can display decimal, hexadecimal and in some cases special characters. Each of the seven LEDs in the display is called a segment, since when it is illuminated, it constructs a segment of the digit, to be displayed. The eighth LED is used for the indication of a decimal point, (DP) and is used when two or more seven-segment displays are put together to display larger numbers.

Each of the seven LEDs in the segment is assigned a label from a through g based on its position in the segment. In addition, one of the LED pins is brought out of the package for external connection, while all other terminals of the LEDs are connected together internally and brought out as single common pin. The common pin is also used to identify the type of the seven-segment display as common anode or common cathode.

- Common cathode: In this display type, all cathode terminals of the LED segments are joined together and are connected to logic low or ground. While based on the number to be displayed a collection of individual segments are illuminated by applying logic high though a current limiting resistor.

- Common anode: In this display type, the connections are reversed. The common terminal is of anode type and is connected with logic high, while individual segment pins are connected to logic low selectively based on the desired digit to be displayed.

The connection labels corresponding to both configurations. Different characters can be displayed by selectively turning on the required LED segments.

Turning ON a specific combination of individual LEDs, one can drive the desired digit pattern on the seven segment LED display. To display the desired digit, an ON/OFF combination of different LEDs is generated. For example, to display the number 2, LED segments a, b, d, e, and g are turned ON. Table provides the display pattern for numbers (0-9) and characters A, b, C, d, E and F for both common anode as well as common cathode type displays.

Time Multiplexing:

Although it is possible to drive each segment of a multi-digit seven-segment display individually with its own dedicated GPIO pins, but the number of port pins required, becomes impractical when more than a few seven-segment displays are involved. To reduce the required number of GPIO pins, it is quite common to use a multiplexed (specifically time multiplexed) display architecture. In a time multiplexed display, each seven segment element is driven individually at a time, but due to the persistence of vision, it appears to the viewer that the entire display is continuously active. It is also important to realize that the implementation for multiplexed seven-segment display is quite straight forward due to the fact that each LED functions as a diode electrically, which conducts in one direction only. If the LED is replaced with a simple light source, implementation for time multiplexed display will become quite difficult.

In an n-digit time multiplexed display, n GPIO pins for digit selection and seven GPIO pins for each of the seven segments (or eight GPIO pins in case, dot point is also used) are required. The resulting total number of GPIO pin connections required is equal to n + 7 (or n + 8). If every segment is interfaced separately to the microcontroller then the number of GPIO pins required is 7n (or 8n). For n = 4, the number of GPIO pins required for individually separate segment connections are 28, while we require only 11 GPIO pins for time multiplexed interfacing. It should be realized that for time multiplexed display each segment is only activated for time fraction of the total display refresh cycle duration. To avoid any flickering effect, the refresh frequency used is approximately 50 Hz in practice. The circuit diagram illustrating the interfacing of four digit seven segment display is shown in Figure. The PNP transistors are used as a switch with the common anode terminals of the display for turning ON one digit at a time. Additionally, the use of these transistors allows higher current sourcing for the common anode terminal.

Task: Code for Seven segment Display

# define SYSCTL_RCGC2_R *(( volatile unsigned long *) 0x400FE108)

# define GPIO_PORTA_DATA_R *(( volatile unsigned long *) 0x400043FC)

# define GPIO_PORTB_DATA_R *(( volatile unsigned long *) 0x400053FC)

# define GPIO_PORTF_DATA_R *(( volatile unsigned long *) 0x40025040)

# define GPIO_PORTA_DIR_R *(( volatile unsigned long *) 0x40004400)

# define GPIO_PORTB_DIR_R *(( volatile unsigned long *) 0x40005400)

# define GPIO_PORTF_DIR_R *(( volatile unsigned long *) 0x40025400)

# define GPIO_PORTA_DEN_R *(( volatile unsigned long *) 0x4000451C)

# define GPIO_PORTB_DEN_R *(( volatile unsigned long *) 0x4000551C)

# define GPIO_PORTF_DEN_R *(( volatile unsigned long *) 0x4002551C)

# define GPIO_PORTF_PUR_R *(( volatile unsigned long *) 0x40025510)

# define SYSCTL_RCGC2_GPIOABF 0x00000023 // Port A,B,F clock en

# define PORT_A_PINS 0x3C // Port A pins 2 ,3 ,4

# define SYSTEM_CLOCK_FREQUENCY 16000000

# define DEALY_DEBOUNCE SYSTEM_CLOCK_FREQUENCY /1000

# define DEALY_DIGIT_REFRESH SYSTEM_CLOCK_FREQUENCY /200

void Port_Init ( void );

void Scan_Switch ( void );

void Binary2BCD ( unsigned int binary_val );

void Delay ( unsigned long counter );

unsigned char Lookup_7Seg_Disp [10] = {0xC0 , 0xF9 , 0xA4 , 0xB0 , 0x92 , 0x82 , 0xF8 , 0x80};

unsigned char Bcd_Val [4] = {0, 0, 0 ,0};

int main ()

{ 	unsigned char i, bcd_digit_value =0;

// Initialize the ports

Port_Init ();

while (1)

{

Scan_Switch ();

for (i = 1; i <= 4; i ++)

{ 	bcd_digit_value = Bcd_Val [4-i];

GPIO_PORTB_DATA_R = Lookup_7Seg_Disp [ bcd_digit_value ];

// Select the digit on Port A

GPIO_PORTA_DATA_R=(PORT_A_PINS - (1 << (i +1)));

Delay ( DEALY_DIGIT_REFRESH );

}

}

}

void Port_Init ()

{

// Enable the clock for port A, B and F

SYSCTL_RCGC2_R |= SYSCTL_RCGC2_GPIOABF ;

// Configure Port A and B direction and digital enable function

GPIO_PORTB_DIR_R = 0xFF ;

GPIO_PORTB_DEN_R = 0xFF ;

GPIO_PORTA_DIR_R = PORT_A_PINS ;

GPIO_PORTF_DIR_R |= 0x10 ;

GPIO_PORTA_DEN_R = PORT_A_PINS ;

GPIO_PORTF_DEN_R |= 0x10 ;

GPIO_PORTF_PUR_R |= 0x10 ;

GPIO_PORTA_DATA_R = PORT_A_PINS ;

GPIO_PORTB_DATA_R = 0xFF ;

}

//time the switch is pressed

void Scan_Switch ( void )

{

static unsigned int count = 0;

static unsigned char flag = 0;

if( GPIO_PORTF_DATA_R == 0)

{

// delay for switch debouncing

Delay ( DEALY_DEBOUNCE );

if (( flag == 0) && ( GPIO_PORTF_DATA_R == 0))

{

count ++;

/* Since we have four digit display , the count value is

reset , if it exceeds 9999. */

if( count >= 9999)

count = 0;

Binary2BCD ( count );

flag = 1;

}

}

else

{

flag = 0;

}

}

void Binary2BCD ( unsigned int binary_val )

{

unsigned int temp ;

temp = binary_val ;

Bcd_Val [3] = temp /1000;

temp = temp - ( Bcd_Val [3] * 1000) ;

Bcd_Val [2] = temp /100;

temp = temp - ( Bcd_Val [2] * 100) ;

Bcd_Val [1] = temp /10;

temp = temp -( Bcd_Val [1] * 10) ;

Bcd_Val [0] = temp ;

}

void Delay ( unsigned long counter )

{

unsigned long i = 0;

for (i =0; i< counter; i ++);

}

Conclusion:

In this lab we did only one task. We used C language in this task on TM4C123G board. With the help of C language in TIVA Board and 7 – Segment Display. First we display just one seven segment display then we use 4 seven segment display and by using C Language code we also displayed the four other seven segment displays.

| NAME |  |
| --- | --- |
| REG #: |  |

EXPERIMENT NO 09:

PWM Signal Generation

Objective:

- Understanding of generation of PWM SIGNAL in Micro-controller.

- How to change the frequency of signal.

- How duty cycle affect the Amplitude.

Apparatus:

- TIVA C Board

- ENERGIA IDE

- Potentio – Meter

- USB Cable

- PC

- Oscilloscope

Theory:

Pulse Width Modulation, or PWM, is a technique for getting analog results with digital means. Digital control is used to create a square wave, a signal switched between on and off. This on-off pattern can simulate voltages in between full on (5 Volts) and off (0 Volts) by changing the portion of the time the signal spends on versus the time that the signal spends off. The duration of "on time" is called the pulse width. To get varying analog values, you change, or modulate, that pulse width. If you repeat this on-off pattern fast enough with an LED for example, the result is as if the signal is a steady voltage between 0 and 5v controlling the brightness of the LED. In the graphic below, the lines represent a regular time period. This duration or period is the inverse of the PWM frequency. A call to analogWrite() is on a scale of 0 - 255, such that analogWrite(255) requests a 100% duty cycle (always on), and analogWrite(127) is a 50% duty cycle. The visual image of PWM can be seen in following figures.

Exercise 1:  (PWM Signal Generate)

Procedure:

- Set up the experiment as shown in the FIGURE 2.

- Connect TIVA C Board to PC using USB Cable.

- Now write code for LED Blinking.

- Additional comand for PWM is used PB_5.

- Then, upload this code to TIVA C Board.

Task 1: Blink led by generating PWM Signal.

Exercise 2: (Change Frequency of Signal)

Procedure:

- Set up the experiment as shown in the FIGURE 2.

- Connect TIVA C Board to PC using USB Cable.

- Now write code for LED Blinking.

- Additional comand for PWM is used PB_5.

- Delay command is used to change frequency.

- We also use delayMicroseconds().

- Then, upload this code to TIVA C Board.

- Repeat this experiment for different delays.

Task 2: Change frequency of PWM signal.

Output:

Conclusion: